Scope of Work

{customerName}

{siteAddress}

Prepared on {today}

Executive Summary

Maryland Smart Energy has conducted a preliminary energy study for {customerName}, identifying the Energy Conservation Measures (ECMs) below. These measures were evaluated based on historical rates of $______ per kWh and $______ per Therm.

Estimated Annual Savings

Energy: ______ kWh/yr

Cost: $______/yr

Proposed ECMs

ECM 1: Optimize HVAC schedule by implementing schedule into thermostats

Optimizing runtime schedules and night setback temperature set points will reduce the total runtime of the units. Unoccupied heating setpoint will be set to 55°F and unoccupied cooling setpoint will be set to 85°F.

Proposed Schedule: ________________________________________

ECM 2: Replace refrigeration temperature sensor

New sensor elements will be installed on walk-in coolers and freezers to enable precise control of food storage temperatures based on the actual temperature of the food products. This improves efficiency by limiting runtime of cooling elements to when it is truly needed.

Proposed Equipment to be Installed:

(____) Networked programmable thermostats

(____) EndoCube Temperature Sensor Elements

ECM Authorization and Refusal Policy

The Customer's authorization to schedule work constitutes full acceptance and approval of all Energy Conservation Measures (ECMs) identified in this document. If the Customer wishes to modify, remove, or dispute any ECM, the Customer must notify Contractor in writing and request a review meeting prior to scheduling. If, after scheduling, the Customer refuses or prevents implementation of any approved ECM at the time of service, such refusal shall be deemed a cancellation of that ECM. The Customer shall be responsible for all costs incurred by Contractor related to that ECM, including but not limited to materials ordered, allocated labor, engineering costs, and administrative costs. Contractor shall issue an invoice for these costs, which shall be due and payable to the contractor within 30 days.

Please sign below to authorize the performance of the scope of work outlined above.

Signature: _________________________________________________

Printed Name: ______________________________________________

Title: _____________________________________________________

Date: ______________________________________________________